⚡ INDUCTION MOTOR HEALTH ASSESSMENT AI

Execution & Testing Guide

Standards: IS 900  |  IEEE 43  |  IS 12075  |  ISO 10816

Internship Project — Actoryx

# PROJECT LOCATION

| Field | Value |
| --- | --- |
| Project Folder | motor_health_ai |
| Full Path | C:\Users\<your-name>\Desktop\motor_health_ai |
| Main File | app.py |
| Standards Engine | motor_checker.py |
| MCP Server | mcp_server.py |
| Web UI | templates\index.html |

# STEP 1 — Open Terminal in Project Folder

- Open File Explorer and navigate to the motor_health_ai folder

- Click on the address bar at the top

- powershellType:

- Press Enter — a terminal opens already inside that folder

# STEP 2 — Activate Virtual Environment

Type this command and press Enter:

.\venv\Scripts\Activate.ps1

If you get a permissions error, run this first:

Set-ExecutionPolicy RemoteSigned -Scope CurrentUser

You know it worked when you see (venv) at the start of the line:

(venv) PS C:\Users\...\motor_health_ai>

# STEP 3 — Install Dependencies (First Time Only)

Run this command once to install all required libraries:

pip install -r requirements.txt

Wait for it to finish. You only need to do this the first time.

# STEP 4 — Start the Web Application

python app.py

Keep this terminal open. The server runs as long as this terminal is open.

You should see:

* Serving Flask app 'app'

* Running on http://127.0.0.1:5000

* Debugger is active!

# STEP 5 — Open the Application in Browser

Open Chrome or Edge and go to:

http://127.0.0.1:5000

You should see the Motor Health Assessment form with 5 input fields.

# STEP 6 — Run the MCP Server (Second Terminal)

Open a NEW terminal window (same trick: address bar → powershell → Enter)

Activate venv again:

.\venv\Scripts\Activate.ps1

Then start the MCP server:

python mcp_server.py

The MCP server exposes your motor assessment as an AI tool that any AI assistant can call.

# STEP 7 — Test with Sample Values

## Test Case 1 — Healthy Motor (All PASS)

| Field | Value to Enter |
| --- | --- |
| Motor Voltage | 415V (Low Tension) |
| IR @ 1 Minute | 3.5 MΩ |
| IR @ 10 Minutes | 5.2 MΩ |
| DC Winding Resistance | 0.051 Ω |
| Vibration | 1.2 mm/sec |

Expected Result: HEALTHY ✅ — All 5 checks PASS. PI = 1.486 (FAIR)

## Test Case 2 — Faulty Motor (Multiple FAIL)

| Field | Value to Enter |
| --- | --- |
| Motor Voltage | 6.6kV (High Tension) |
| IR @ 1 Minute | 4.0 MΩ |
| IR @ 10 Minutes | 4.2 MΩ |
| DC Winding Resistance | 0.060 Ω |
| Vibration | 4.5 mm/sec |

Expected Result: CRITICAL ❌ — IR, PI, DC Resistance and Vibration all FAIL

# Standards Reference — IS 900 / IEEE 43

| # | Check | Standard | 415V Limit | 6.6kV Limit |
| --- | --- | --- | --- | --- |
| I | IR @ 1 min | IS 900 / IEEE 43 | >= 1.5 MΩ | >= 6.5 MΩ |
| II | IR @ 10 min | IS 900 / IEEE 43 | >= 1.5 MΩ | >= 6.5 MΩ |
| III | Polarisation Index (PI) | IEEE 43 | PI >= 1.2 | PI >= 1.2 |
| IV | DC Winding Resistance | Design Spec | 0.045 – 0.055 Ω | 0.045 – 0.055 Ω |
| V | Vibration | IS 12075/ISO 10816 | < 3.0 mm/sec | < 3.0 mm/sec |

# Quick Reference — Ports & URLs

| Service | URL |
| --- | --- |
| Web Application | http://127.0.0.1:5000 |
| MCP Server | stdio (runs in terminal) |
| Flask Debug | http://127.0.0.1:5000 |

# Project File Structure

motor_health_ai/

├── app.py              ← Flask web server + diagnostic report

├── motor_checker.py    ← IS 900 / IEEE 43 standards engine

├── mcp_server.py       ← MCP server (exposes tool to AI)

├── mcp_config.json     ← MCP configuration

├── requirements.txt    ← Python dependencies

├── .gitignore          ← Git ignore rules

└── templates/

└── index.html      ← Web UI frontend

# To Stop the Application

Go back to the terminal where app.py is running and press:

Ctrl + C

This stops the Flask server. To restart, run python app.py again.

Induction Motor Health Assessment AI  |  Actoryx Internship Project  |  IS 900 / IEEE 43